Task 3.3: Firebase Cloud Logging — Continuation of Task 3.1

Submission Format: GitHub repo link + Firebase console screenshots + short screen recording

Goal: Extend the Node-RED system from Task 12 so live sensor data and motor state also land in Firebase — giving the system a cloud record on top of the local dashboard.

Part 1: Firebase Setup

- Create a Firebase project and a Realtime Database. Test mode rules are fine for this task.

- Install the Firebase nodes for Node-RED .

Part 2: Wire It Into the Existing Flow

- Don't duplicate logic. Task 12's flow already receives BME280 data and motor state over MQTT — tap off that same flow and add a Firebase-write branch, rather than building a second parallel system.

- Live/current state: temperature, humidity, pressure, motor running/stopped, direction, speed — overwritten on every update, same node in Firebase always reflects "right now."

- History log: append a timestamped record on an interval (10–30s, their choice, documented) so there's a record over time, not just the latest snapshot.

Part 3: Requirements

- Firebase writes must not block or slow down the live Node-RED dashboard from Task 12 — it keeps working exactly as before, Firebase is additive.

- Reuse the LWT/offline detection already built in Task 12 — when the ESP32 goes offline, that state should also show in Firebase, not freeze on stale "online" data.

- Sensor values written to Firebase must be the same validated/clamped values shown on the dashboard — no raw/unvalidated data going to the cloud separately.

Definition of Done

- [ ] Firebase Realtime Database created and receiving writes from Node-RED.

- [ ] Current sensor + motor state visible live in the Firebase console, matching the Node-RED dashboard in real time.

- [ ] Historical entries accumulate over time with timestamps, at the documented interval.

- [ ] Disconnecting the ESP32 (WiFi or power) reflects as offline in Firebase within a few seconds — same LWT signal used in Task 12.

- [ ] Updated Node-RED flow exported (flows.json) included in repo, alongside the Task 12 version.

- [ ] README explains the data structure (current vs. history) and how the Firebase branch was added onto the existing flow without duplicating logic.