T E C H N I C A L   R E P O R T

IoT / EMBEDDED SYSTEMS

FreeRTOS Queue-Based Sensor Data Transfer - Architectural & Implementation

Engineering Document

Daily Technical Report, Day 35

| Project | Queue-Based Producer-Consumer Communication Between ESP32 Tasks |
| --- | --- |
| Target MCU | ESP32 Development Board running ESP-IDF / FreeRTOS |
| Input Signals | Potentiometer ADC reading + digital IR state |
| Output Signals | PWM LED brightness + digital LED state |
| Inter-Task Transport | FreeRTOS Queue using sensor_message_t copies |
| Queue Experiment | Depth 1 first, then depth 5; producer intentionally faster than consumer |
| Execution Status | Hardware operational; queue pipeline verified; timing/overflow test in progress |
| Prepared By | Malak Alsharqawi |
| Prepared For | IOT Training Program |

Confidential, Internal Training Documentation

Contents

| 1 | Daily Overview |
| --- | --- |
| 1.1 | Summary |
| 1.2 | Goal & Submission Scope |
| 1.3 | Tasks Completed |
| 2 | Conceptual Foundation: Why the Queue Replaces Shared Globals |
| 2.1 | Message Passing Instead of Shared Sensor State |
| 2.2 | Exact Task 3.6 Race Condition |
| 2.3 | Why a Global Struct Alone Would Not Solve It |
| 2.4 | Queue Message Passing and Data Ownership |
| 2.5 | What the Queue Contains Under the Hood |
| 2.6 | Blocking vs. Polling |
| 2.7 | Full and Empty Queue Symmetry |
| 2.8 | Queue Depth, Throughput, Backpressure, and Data Loss |
| 2.9 | Stale Queue Data vs. Stale Shared-Variable Data |
| 2.10 | Queue vs. Mutex vs. Semaphore |
| 3 | System Architecture & Concurrency Model |
| 3.1 | Producer-Consumer Ownership Model |
| 3.2 | Scheduler Blocking Model |
| 4 | Hardware Test Setup & Schematic |
| 5 | Queue Design and API Configuration |
| 5.1 | sensor_message_t and QueueHandle_t |
| 5.2 | xQueueCreate() |
| 5.3 | xQueueSend() |
| 5.4 | xQueueReceive() |
| 6 | Implementation Breakdown |
| 6.1 | Sensor Task |
| 6.2 | Output Task |
| 6.3 | app_main() Initialization Order |
| 7 | Queue Depth Experiment & Timing Analysis |
| 7.1 | Depth 1 |
| 7.2 | Depth 5 |
| 7.3 | Stale Data and Latency |
| 8 | Troubleshooting Log |
| 9 | Verification & Requirements Compliance |
| 10 | Definition of Done |
| 11 | Next Steps |
| 12 | Appendix: Core Implementation Snippets |

# 1. Daily Overview

## 1.1 Summary

This task upgrades the previous two-task ESP32 design from unsynchronized shared global variables to explicit FreeRTOS message passing. The Sensor Task reads the potentiometer and digital IR input, packages both values into one sensor_message_t structure, and sends a complete copy through a FreeRTOS Queue. The Output Task blocks on the queue, receives its own copy, and then updates PWM brightness and the digital LED output.

The core engineering objective is not only to make the hardware work, but to prove that the new architecture solves the specific Task 3.6 race condition: the consumer must no longer observe an ADC value from one sampling moment and an IR value from another. The queue turns the two values into one atomic application-level message and also provides synchronization with the FreeRTOS scheduler.

## 1.2 Goal & Submission Scope

Goal: Replace the Task 3.6 shared sensor globals with a proper FreeRTOS Queue and experimentally verify queue-full behavior at two queue depths.

Use the same Sensor Task and Output Task roles from the previous design.

Define one message structure containing potentiometer value and IR state.

Pass all sensor data through xQueueCreate(), xQueueSend(), and xQueueReceive().

Sensor Task sends on a fixed interval; Output Task blocks on xQueueReceive() instead of polling.

Test queue depth 1 first, then depth 5, while the producer runs faster than the consumer.

Log every send and receive using ESP_LOG and use the logs as experimental evidence.

## 1.3 Tasks Completed

Removed direct shared adc_raw and ir_state communication from the task interface.

Defined sensor_message_t to carry a coherent sensor snapshot.

Created a QueueHandle_t and initialized the queue before task creation.

Moved xQueueSend() inside the Sensor Task loop so every sensor sample can actually be transmitted.

Configured the Output Task to block on xQueueReceive() and update PWM / LED outputs only after a valid message is received.

Validated the hardware path after the queue integration; a small visible latency remains when deliberate processing delays are enabled.

# 2. Conceptual Foundation: Why the Queue Replaces Shared Globals

The previous implementation stored adc_raw and ir_state as global variables. The Sensor Task wrote them independently, while the Output Task read them independently. Because FreeRTOS may context-switch between normal C statements, the consumer could observe a logically inconsistent pair even if each individual integer access was valid.

// Previous shared-memory pattern int adc_raw; int ir_state; // Sensor Task adc_raw = new_adc; ir_state = new_ir; // Output Task use(adc_raw, ir_state);

Race condition: A context switch can occur after adc_raw is updated but before ir_state is updated. The Output Task can therefore read a new ADC value together with an old IR value.

## 2.1 Message Passing Instead of Shared Sensor State

typedef struct { int adc_raw; int ir_state; } sensor_message_t;

The Sensor Task now creates a complete local snapshot before sending. FreeRTOS copies that fixed-size structure into queue-managed storage. The Output Task later receives another copy into its own local variable. This separates ownership cleanly: producer local memory -> queue storage -> consumer local memory.

| Design Property | Shared Globals | FreeRTOS Queue |
| --- | --- | --- |
| Data ownership | Both tasks touch the same variables | Each task owns its local copy |
| Snapshot consistency | Can mix values from different moments | ADC and IR travel together as one message |
| Waiting strategy | Usually polling or timed reads | Consumer can block efficiently |
| Buffering | No history; only current variables | Temporary FIFO buffer with defined depth |
| Overflow behavior | Not explicit | Controlled by queue depth and send timeout |

## 2.2 The Exact Task 3.6 Race Condition - An Inconsistent Snapshot

The original problem was not simply that two integers were global. The real concurrency problem was that adc_raw and ir_state represented one logical sensor sample but were updated as two separate memory operations. FreeRTOS is allowed to schedule another ready task between those statements. The Output Task could therefore read a new ADC value together with an old IR value and treat that mixed pair as if it came from one sampling instant.

Sensor Task Output Task ----------- ----------- adc_raw = 2500 ¦ ¦ context switch +--------------------------> reads adc_raw = 2500 reads ir_state = 0 <- old sample <-------------------------- context switch ir_state = 1 Intended sample: {2500, 1} Observed sample: {2500, 0}

Important distinction: a single integer read/write may be atomic on the ESP32, but that does not make a multi-field sensor sample atomic as a whole. The Queue solves the logical snapshot problem by transferring the fields together.

## 2.3 Why a Global Struct Alone Would Not Solve It

Packing the two globals into one struct improves organization, but it does not automatically add synchronization. If that struct remains global and the Sensor Task updates its members one after another, a context switch can still occur between the member assignments. The Output Task can still observe a partially updated structure.

// Better organization, but still shared mutable memory typedef struct { int adc_raw; int ir_state; } sensor_message_t; sensor_message_t shared_data; shared_data.adc_raw = 2500; /* scheduler may switch tasks here */ shared_data.ir_state = 1;

This is why the design change is not merely 'put the values in a struct'. The real change is from shared mutable state to message passing: the producer first builds a complete local struct and only then sends one complete item.

## 2.4 Queue Message Passing and Data Ownership

With the Queue design, each stage owns a different copy of the message. The Sensor Task owns sensor_data while it is sampling. xQueueSend() copies that item into FreeRTOS-managed Queue storage. Later, xQueueReceive() copies the oldest queued item into received_data owned by the Output Task. The two tasks no longer need to read and write the same sensor payload in RAM.

Sensor Task local memory +---------------------------+ ¦ sensor_data ¦ ¦ adc_raw = 2500 ¦ ¦ ir_state = 1 ¦ +-------------+-------------+ ¦ ¦ xQueueSend() copies sizeof(sensor_message_t) v +---------------------------+ ¦ FreeRTOS Queue storage ¦ ¦ {2500, 1} ¦ +-------------+-------------+ ¦ ¦ xQueueReceive() copies and removes oldest item v +---------------------------+ ¦ Output Task local memory ¦ ¦ received_data = {2500, 1} ¦ +---------------------------+

A successful send therefore does not mean the Output Task has already processed the message. It only means that a copy has entered the Queue. Sampling, enqueueing, waiting, receiving, and processing are separate stages.

## 2.5 What the Queue Contains Under the Hood

Conceptually, a FreeRTOS Queue is more than an array. It combines fixed-size storage with metadata and scheduler integration. FreeRTOS must know the item size, maximum depth, number of messages currently waiting, read/write positions, and which tasks are blocked waiting to send or receive.

+------------------------------------------------+ ¦ Queue control information ¦ +------------------------------------------------+ ¦ item size = sizeof(sensor_message_t) ¦ ¦ maximum length = QUEUE_DEPTH ¦ ¦ messages currently waiting ¦ ¦ read position / write position ¦ ¦ tasks waiting because queue is empty ¦ ¦ tasks waiting because queue is full ¦ +------------------------------------------------+ ¦ Fixed-size message storage (circular style) ¦ +------------------------------------------------+

The storage behaves conceptually like a circular buffer. When an item is received, FreeRTOS advances its read position and frees one slot; when an item is sent, it advances the write position. Items do not need to be shifted through memory after every receive.

## 2.6 Blocking vs. Polling - Why the Queue Also Synchronizes Tasks

The Queue is not only a data container; it is also a synchronization point. With polling, the Output Task would wake repeatedly just to ask whether new data exists. With a blocking receive, FreeRTOS can remove the Output Task from CPU scheduling until either a message arrives or its timeout expires.

Polling approach: RUNNING -> check -> no data -> delay/check -> no data -> check ... Blocking Queue approach: RUNNING -> xQueueReceive() ¦ +-- queue has data -> receive immediately ¦ +-- queue empty -> BLOCKED ¦ producer sends message ¦ v READY ¦ scheduler selects task ¦ v RUNNING

A blocked task does not burn CPU cycles waiting. This is different from vTaskDelay(): a delay sleeps for a fixed time even if data arrives early, while xQueueReceive() can wake as soon as the event it is waiting for occurs.

## 2.7 Full and Empty Queue Symmetry

The send and receive sides have a useful symmetry. A receiver may block because the Queue is empty; a sender may block because the Queue is full. Sending an item can wake a waiting receiver, while receiving an item can free a slot and wake a waiting sender.

QUEUE EMPTY ¦ receiver calls xQueueReceive(wait > 0) ¦ receiver -> BLOCKED ¦ producer sends ¦ receiver -> READY QUEUE FULL ¦ sender calls xQueueSend(wait > 0) ¦ sender -> BLOCKED ¦ consumer receives ¦ sender -> READY

In this assignment, a zero-wait send is especially useful for the overflow experiment because it makes queue-full behavior explicit: the send fails immediately, the existing queued item remains, and the new reading is not stored.

## 2.8 Queue Depth, Throughput, Backpressure, and Data Loss

Queue depth controls how much temporary mismatch between producer and consumer rates can be absorbed. Depth 1 can hold only one waiting message. Depth 5 can absorb a larger burst, but neither depth changes how fast the Output Task actually processes data. If the producer remains permanently faster, every finite Queue will eventually fill.

Zero-wait send on a full Queue: the new message is rejected immediately; the old queued message is not overwritten.

Nonzero send timeout on a full Queue: the Sensor Task may enter the Blocked state while waiting for space.

Blocking a producer creates backpressure: the slow consumer indirectly slows future production.

A larger Queue reduces immediate overflow during bursts but consumes more RAM and may increase latency.

## 2.9 Stale Queue Data vs. Stale Shared-Variable Data

The word stale means something different after moving to a Queue. A queued item can be perfectly valid and internally consistent but old relative to the current physical input because newer samples are waiting behind it. That is Queue latency. In the old global-variable design, stale data could instead mean that the consumer reread an old value without knowing whether a new sample had arrived, and it could also combine values from different sampling moments.

Current potentiometer position -> 3000 Queue backlog: Front v [1200] [1600] [2100] [2600] [3000] ^ newest xQueueReceive() returns 1200 first. 1200 is valid, but stale relative to the current input.

This is why a deeper Queue is not automatically 'better'. If every sample matters, buffering history may be correct. If only the latest state matters, carrying a long backlog can make the physical response feel slow.

## 2.10 Queue vs. Mutex vs. Semaphore

FreeRTOS provides several synchronization primitives, but they solve different problems. This task uses a Queue because data itself must move from one task to another.

| Primitive | Primary purpose | What it would mean in this task |
| --- | --- | --- |
| Queue | Transfer messages + synchronize | Send ADC/IR snapshot from Sensor Task to Output Task |
| Mutex | Protect shared resource | Would protect shared memory, but would not itself transport the sample |
| Semaphore | Signal event / resource count | Could signal that something happened, but carries little/no payload |

Mental model: Sensor Task creates one complete snapshot -> xQueueSend() copies it into Queue storage -> Output Task blocks until data exists -> xQueueReceive() copies and removes the oldest item -> Output Task owns and processes that copy.

# 3. System Architecture & Concurrency Model

The firmware is organized as a simple producer-consumer pipeline. Hardware acquisition and hardware actuation remain in separate tasks; the FreeRTOS Queue is the only sensor-data path between them.

[ Potentiometer + IR Input ] ¦ v [ Sensor Task ] local sensor_message_t ¦ xQueueSend() ¦ v [ sensor_queue ] FIFO + blocking ¦ xQueueReceive() ¦ v [ Output Task ] local received_data copy ¦ v [ PWM LED + LED ]

## 3.1 Producer-Consumer Ownership Model

Sensor Task is the producer: it acquires ADC and IR values and creates one complete message.

sensor_queue is the communication and synchronization object managed by FreeRTOS.

Output Task is the consumer: it receives a complete message, updates PWM duty, and sets the digital LED state.

## 3.2 Scheduler Blocking Model

If the queue is empty, xQueueReceive() can move the Output Task to the Blocked state. A blocked task does not continuously consume CPU time. When the Sensor Task sends a message, FreeRTOS can move the receiver back to Ready and schedule it according to task priority.

Queue empty -> Receiver may BLOCK Send succeeds -> Waiting receiver becomes READY Queue full -> Sender may BLOCK or FAIL, depending on xTicksToWait Receive succeeds -> One slot becomes free

# 4. Hardware Test Setup & Schematic

The provided CirkIt Designer schematic documents the bench wiring used to verify the queue-controlled firmware. It shows the ESP32 development board, potentiometer input wiring, breadboard power distribution, and red/green LED output connections. The firmware additionally reads a digital IR state; a discrete IR sensor module is not visibly represented in this supplied schematic, so this figure is treated as the provided bench wiring reference rather than a complete sensor inventory.

Figure 1. Provided CirkIt Designer schematic for the ESP32 queue-task hardware test setup.

| Hardware Element | Role in Firmware |
| --- | --- |
| ESP32 | Runs ESP-IDF / FreeRTOS, ADC acquisition, queue logic, PWM, and GPIO control |
| Potentiometer | Analog input used to generate adc_raw for PWM control |
| Red / Green LEDs | Visual output indicators driven by PWM and digital GPIO logic |
| Breadboard / jumpers | Power and signal interconnect for the bench test |
| IR digital state | Read by firmware and included in sensor_message_t; sensor module is not shown in the provided schematic |

# 5. Queue Design and API Configuration

## 5.1 sensor_message_t and QueueHandle_t

typedef struct { int adc_raw; int ir_state; } sensor_message_t; static QueueHandle_t sensor_queue;

sensor_message_t defines the application-level protocol between the two tasks. QueueHandle_t is an opaque reference to the FreeRTOS queue object; it is not the sensor data itself.

## 5.2 xQueueCreate()

sensor_queue = xQueueCreate( QUEUE_DEPTH, sizeof(sensor_message_t) );

The first argument is the maximum number of messages that may wait simultaneously. The second argument is the fixed byte size of every queue item. The queue is created before the tasks so neither task can access an uninitialized handle.

## 5.3 xQueueSend()

if (xQueueSend(sensor_queue, &sensor_data, 0) == pdPASS) { ESP_LOGI(SENSOR_TAG, "Sensor data sent"); } else { ESP_LOGW(SENSOR_TAG, "Queue full - send failed"); }

xQueueSend() copies sizeof(sensor_message_t) bytes from &sensor_data into queue storage. With zero wait, a full queue causes immediate failure and the new reading is not stored. This makes overflow behavior easy to observe in the experiment.

## 5.4 xQueueReceive()

if (xQueueReceive(sensor_queue, &received_data, portMAX_DELAY) == pdTRUE) { // received_data now contains the oldest queued message }

xQueueReceive() copies the oldest FIFO message into received_data and removes that item from the queue. portMAX_DELAY makes the consumer block efficiently while the queue is empty.

# 6. Implementation Breakdown

## 6.1 Sensor Task - Producer

void sensor_task(void *pvParameters) { sensor_message_t sensor_data; while (1) { adc_oneshot_read(adc1_handle, POT_CHANNEL, &sensor_data.adc_raw); sensor_data.ir_state = gpio_get_level(IR_GPIO); xQueueSend(sensor_queue, &sensor_data, 0); vTaskDelay(pdMS_TO_TICKS(SENSOR_INTERVAL_MS)); } }

The critical implementation rule is that xQueueSend() must be inside the infinite loop. In an earlier version it was placed after while(1), which made the send code unreachable. The Sensor Task therefore read hardware continuously but never placed a message in the queue.

## 6.2 Output Task - Consumer

void output_task(void *pvParameters) { sensor_message_t received_data; while (1) { if (xQueueReceive(sensor_queue, &received_data, portMAX_DELAY) == pdTRUE) { ledc_set_duty(LEDC_LOW_SPEED_MODE, LEDC_CHANNEL_0, received_data.adc_raw); ledc_update_duty(LEDC_LOW_SPEED_MODE, LEDC_CHANNEL_0); gpio_set_level(BLINK_GPIO, !received_data.ir_state); } } }

The Output Task uses only received_data. It no longer reads producer-owned sensor globals, so the specific Task 3.6 shared-data race is removed.

## 6.3 app_main() Initialization Order

setup(); sensor_queue = xQueueCreate(QUEUE_DEPTH, sizeof(sensor_message_t)); if (sensor_queue == NULL) return; xTaskCreate(sensor_task, "Sensor Task", 2048, NULL, 1, NULL); xTaskCreate(output_task, "Output Task", 2560, NULL, 2, NULL);

# 7. Queue Depth Experiment & Timing Analysis

The experiment deliberately configures the Sensor Task to produce messages faster than the Output Task consumes them. This exposes overflow and latency behavior instead of hiding it behind a perfectly balanced timing configuration.

## 7.1 Depth 1

QUEUE_DEPTH = 1 Sensor interval = 100 ms Output processing time = 500 ms Send wait = 0 ticks

0 ms SEND A -> success ~0 ms RECEIVE A -> success; consumer begins processing 100 ms SEND B -> success (one queue slot available) 200 ms SEND C -> fail: queue still contains B 300 ms SEND D -> fail 400 ms SEND E -> fail 500 ms consumer ready -> RECEIVE B

Depth 1 provides almost no burst buffering. It produces low queueing latency for messages that are accepted, but new readings are easily dropped when the consumer is deliberately slowed.

## 7.2 Depth 5

Changing only QUEUE_DEPTH from 1 to 5 allows several readings to accumulate while the consumer is busy. Fewer immediate send failures are expected, but the consumer may process older readings before reaching the newest value.

| Metric | Depth 1 | Depth 5 |
| --- | --- | --- |
| Burst capacity | 1 waiting message | 5 waiting messages |
| Overflow | Occurs quickly | Delayed |
| Dropped readings | More likely with zero-wait send | Less likely during short bursts |
| Latency / backlog | Usually smaller | Can grow as queue fills |
| Best observation | Clear overflow behavior | Buffering vs stale-data trade-off |

## 7.3 Stale Data and Latency

A deeper queue does not make the consumer faster. If the producer remains faster for long enough, any finite queue eventually fills. A queued message may also become stale: it can be internally correct and coherent, yet old relative to the current physical potentiometer position.

# 8. Troubleshooting Log

| Issue | Root Cause | Resolution |
| --- | --- | --- |
| xTaskCreate incompatible pointer type | String tag such as SENSOR_TASK was passed as the task function argument | Pass sensor_task / output_task as the first argument; keep text strings only for task names or log tags |
| Hardware outputs did not update after queue integration | xQueueSend() was placed after while(1), making the send path unreachable | Move xQueueSend() inside the Sensor Task loop after both sensor values are acquired |
| Output Task appeared frozen | xQueueReceive(..., portMAX_DELAY) correctly blocked forever because no producer message was ever sent | Once xQueueSend() ran inside the loop, the receiver woke and hardware updated |
| Hardware response felt slightly slow | Deliberate output processing delay and/or queue backlog introduced visible latency | Keep the delay for overflow testing; remove or reduce it for normal responsive operation |
| Potential overflow ambiguity | Nonzero send timeout can block the producer and hide immediate overflow | Use xQueueSend(..., 0) for the depth-comparison experiment to make full-queue failures explicit |

Key debugging lesson: With RTOS code, a function can compile correctly and still never execute. Control-flow placement inside or outside an infinite task loop is as important as the API call itself.

# 9. Verification & Requirements Compliance

The hardware is now operating with the queue-based task pipeline. The user observed a small amount of slowness, which is consistent with the intentionally slow consumer configuration used to create queue pressure. A serial monitor log was not supplied with this report, so exact counts of successful sends, failed sends, and receive timestamps are not claimed here.

| Requirement | How It Is Satisfied | Status |
| --- | --- | --- |
| Same Sensor and Output task roles | Existing two-task architecture retained | Met |
| Struct message containing ADC + IR | sensor_message_t contains adc_raw and ir_state | Met |
| xQueueCreate / xQueueSend / xQueueReceive | All three APIs are integrated in the design | Met |
| No shared sensor globals | Output consumes received_data rather than direct sensor globals | Met in queue path |
| Output blocks instead of polling | xQueueReceive(..., portMAX_DELAY) blocks on empty queue | Met |
| Depth 1 and depth 5 comparison | QUEUE_DEPTH is designed to be changed between the two experimental runs | Ready / requires captured logs for final evidence |
| ESP_LOG on send and receive | Logging statements included in producer and consumer paths | Met in implementation |
| Serial Monitor evidence | Exact log capture not included in the material supplied for this report | Pending capture |

# 10. Definition of Done

| Criterion | Status |
| --- | --- |
| Global adc_raw / ir_state communication removed from the task data path | Met |
| Queue created before task execution | Met |
| Sensor Task sends one coherent message per sample cycle | Met |
| Output Task blocks on queue and updates hardware only after receive | Met |
| Depth 1 overflow behavior captured in log | Pending final Serial Monitor capture |
| Depth 5 comparison captured with same timing | Pending final Serial Monitor capture |
| Technical report explains race-condition change and stale-data trade-off | Met |

# 11. Next Steps

Run the depth-1 configuration with Sensor Task faster than Output Task and capture at least 10-20 seconds of Serial Monitor logs.

Change only QUEUE_DEPTH to 5 and repeat the test with identical producer and consumer timing.

Count or visually compare SEND OK, SEND FAILED, and RECEIVE OK events between the two runs.

Document whether the larger queue reduced drops but increased visible latency or backlog.

After the experiment, remove the deliberate Output Task processing delay for normal responsive hardware operation.

Optionally add uxQueueMessagesWaiting(sensor_queue) to diagnostic logs to show queue occupancy during the experiment.

Commit the final source code, Serial Monitor evidence, schematic, and report to the training repository.

## 11.1 Expected Final Engineering Conclusion

The FreeRTOS Queue solves the specific Task 3.6 race condition because the Sensor Task and Output Task no longer directly exchange data through independently updated global sensor variables. Instead, the producer creates one coherent snapshot, FreeRTOS copies it into synchronized queue storage, and the consumer later receives its own copy. Queue depth does not eliminate throughput mismatch: depth 1 exposes overflow quickly, while depth 5 buffers a larger burst at the cost of additional backlog and potential stale-data latency.

# 12. Appendix: Core Implementation Snippets

## 12.1 Queue Definitions

typedef struct { int adc_raw; int ir_state; } sensor_message_t; static QueueHandle_t sensor_queue;

## 12.2 Producer Send Path

sensor_message_t sensor_data; while (1) { ret = adc_oneshot_read(adc1_handle, POT_CHANNEL, &sensor_data.adc_raw); sensor_data.ir_state = gpio_get_level(IR_GPIO); if (xQueueSend(sensor_queue, &sensor_data, 0) != pdPASS) ESP_LOGW(SENSOR_TAG, "Queue full - send failed"); else ESP_LOGI(SENSOR_TAG, "SEND OK: adc_raw=%d, ir_state=%d", sensor_data.adc_raw, sensor_data.ir_state); vTaskDelay(pdMS_TO_TICKS(SENSOR_INTERVAL_MS)); }

## 12.3 Consumer Receive Path

sensor_message_t received_data; while (1) { if (xQueueReceive(sensor_queue, &received_data, portMAX_DELAY) == pdTRUE) { ledc_set_duty(LEDC_LOW_SPEED_MODE, LEDC_CHANNEL_0, received_data.adc_raw); ledc_update_duty(LEDC_LOW_SPEED_MODE, LEDC_CHANNEL_0); gpio_set_level(BLINK_GPIO, !received_data.ir_state); } }

## 12.4 Queue Initialization

sensor_queue = xQueueCreate(QUEUE_DEPTH, sizeof(sensor_message_t)); if (sensor_queue == NULL) { ESP_LOGE("MAIN", "Failed to create sensor queue"); return; } xTaskCreate(sensor_task, "Sensor Task", 2048, NULL, 1, NULL); xTaskCreate(output_task, "Output Task", 2560, NULL, 2, NULL);

End of Technical Report