IoT / EMBEDDED SYSTEMS

Daily Technical Report

| Project Title | IoT Training Program |
| --- | --- |
| Report / Day No. | Day 1 |
| Date | 19/7/2026 |
| Prepared By | Malak Alsharqawi |
| Organization | Cyber Robot |

Confidential — Internal Training Documentation

# Table of Contents

Table of Contents	2

1. Daily Overview	3

1.1 Summary	3

1.2 Tasks Completed Today	3

Task 1:	3

1. Objective	3

2. Procedure	3

Step 1 — Install the CP210x USB-to-UART driver	3

Step 2 — Install Windows Subsystem for Linux (WSL)	3

Step 3 — Install and configure PlatformIO	3

Step 4 — Configure USB/serial passthrough (usbipd)	3

3. Code / Configuration	4

4. Schematic	4

5. Photos	5

6. Issues & Troubleshooting	5

7. Results & Outcome	5

Task 2: GitHub	5

1. Objective	5

2. Procedure	5

Phase 1 — SSH key setup	5

Phase 2 — Initialize and link the repository	5

Phase 3 — Push to GitHub	6

3. Code / Configuration	6

4. Issues & Troubleshooting	6

5. Results & Outcome	6

3. Project Documentation & Notes	6

# 1. Daily Overview

## 1.1 Summary

This session successfully set up a complete ESP32 development environment by linking Windows hardware to a Linux-based WSL backend for building and flashing firmware. The environment was fully verified by running an external LED blink sketch and establishing a secure, password-free connection to GitHub using SSH.

## 1.2 Tasks Completed Today

- VS Code & PlatformIO Setup + ESP32 Blink

- Git & Github

# Task 1: VS Code & PlatformIO Setup + ESP32 Blink

## 1. Objective

Set up a complete embedded development environment capable of recognizing, building for, and flashing firmware to an ESP32 microcontroller from a Linux-based toolchain running on Windows.

## 2. Procedure

### Step 1 — Install the CP210x USB-to-UART driver

- Downloaded and installed the CP210x VCP driver so Windows recognizes the ESP32 over USB.

### Step 2 — Install Windows Subsystem for Linux (WSL)

- Ran the one-command installation and restarted the machine to apply virtualization layers.

- Initialized the Linux user account (username and password) on first boot.

- Verified the installed distribution and WSL version using the status list command.

- Updated the internal package database before installing any compilers or extensions.

### Step 3 — Install and configure PlatformIO

- Installed the PlatformIO IDE extension inside VS Code (connected to the WSL: Ubuntu backend).

- Resolved a Python interpreter error by installing python3 and python3-venv inside WSL, then retried the extension setup.

### Step 4 — Configure USB/serial passthrough (usbipd)

- Granted the Linux user permission to access serial devices (dialout group).

- Force-bound the ESP32's USB Bus ID on the Windows host using usbipd.

- Attached the device to the WSL distribution with auto-attach enabled for persistent reconnection.

## 3. Code / Configuration

Key commands used during setup:

# Install WSL wsl --install # Verify installation wsl --list -v # Update Linux packages sudo apt update && sudo apt upgrade -y # Fix PlatformIO Python interpreter error sudo apt update && sudo apt install python3 python3-venv -y # Grant serial device permissions sudo usermod -a -G dialout $USER # Bind and attach ESP32 (run in Windows PowerShell as Administrator) usbipd list usbipd bind --busid <YOUR_BUS_ID> --force usbipd attach --wsl --busid <YOUR_BUS_ID> --auto-attach

## 4. Schematic

## 5. Photos

## 6. Issues & Troubleshooting

PlatformIO initially failed with “Can not find working Python 3.6+ Interpreter” because it was searching for Python inside the WSL Linux environment rather than Windows. Resolved by installing python3 and python3-venv inside WSL and retrying.

## 7. Results & Outcome

The ESP32 was successfully recognized by WSL over a shared USB/serial connection, PlatformIO built and uploaded a basic Blink LED sketch (GPIO 18, 115200 baud), and serial monitor output was confirmed to display “External LED is ON/OFF” at 1-second intervals.

# Task 2: GitHub

## 1. Objective

Turn an existing local project folder into a version-controlled Git repository and link it to a remote GitHub repository using a secure SSH connection.

## 2. Procedure

### Phase 1 — SSH key setup

- Generated a new ed25519 SSH key pair and added the public key to GitHub (Settings → SSH and GPG keys).

### Phase 2 — Initialize and link the repository

- Navigated to the project directory and ran git init to start version tracking.

- Staged and committed all existing files as the initial commit.

- Renamed the default branch to main and added the GitHub remote using the SSH URL.

### Phase 3 — Push to GitHub

- Pushed the local commit history to GitHub and set the upstream tracking branch.

- Accepted GitHub's host fingerprint on first connection.

## 3. Code / Configuration

Commands executed:

# Generate SSH key ssh-keygen -t ed25519 -C "your_email@example.com" cat ~/.ssh/id_ed25519.pub # copy this into GitHub # Initialize and commit cd ~/test/IoT_training git init git add . git commit -m "Initial commit of existing project files" git branch -M main git remote add origin git@github.com:<user>/<repo>.git # Push git push -u origin main

## 4. Issues & Troubleshooting

None — first-time SSH host verification prompt was accepted as expected.

## 5. Results & Outcome

The local project was successfully version-controlled and pushed to the remote GitHub repository over SSH; subsequent commits can now be pushed directly with git push.

# 3. Project Documentation & Notes

- Development Notes & Key Learnings All daily progression logs, troubleshooting steps, and core takeaways for this initial project milestone are actively documented here: 👉 View My First-Day Notion Notebook

- Hardware Schematic & Circuit Layout The system architecture design, GPIO pinout mapping, and electrical connections were mapped out using the cloud platform: 👉 Open Circuit Designer Workspace

- GPIO Pinout

End of Report